COMMAND VEHICLE CHECKLIST

INDEX

1.  INITIAL WALKAROUND

2.  INITIAL SETUP AT SITE

3. VEHICLE STARTUP

4. POWER-UP SECTION

* power up with generator

* power up with external power

5. STABALIZER JACK OPERATION

Stabalizer jack extension

Stabalizer jack retraction

6. MAST OPERATION

Mast extension

Mast retraction

7. POWER-DOWN OPERATION

8. PRIOR TO DEPARTURE FROM SITE

COMMAND VEHICLE CHECKLIST

INITIAL WALKAROUND

1. Mast Retracted – CHECKED

2. Mast warning actuator pin – INSTALLED

3. Stablalizer jacks stowed - CHECKED

4. Chocks – REMOVED

5. Obstructions (above, below, front, rear) - CHECKED

COMMAND VEHICLE CHECKLIST

VEHICLE STARTUP

COMMAND VEHICLE CHECKLIST

INITIAL SETUP AT SITE

1. Vehicle position checked for overhead and underside obstructions - CHECKED

2. Vehicle correctly aligned - COMPLETED

3. Cameras correctly aligned - COMPLETED

4. Chocks – INSTALLED

5. Main door – UNLOCKED/OPEN

6. Ladder – INSTALLED

7. Mast compartment door – UNLOCKED/OPEN

8. Roof ladder - REMOVED

9. Generator compartment door - UNLOCKED/OPEN

COMMAND VEHICLE CHECKLIST

# POWER UP with GENERATOR

1. Generator Oil Level Checked

2. Air inlets and outlets clear

3. Power selector switch – OFF

4. Generator start/stop switch – START (Hold until     generator starts and comes up to speed) (10 seconds  max – allow 60 second cool down betwee4en  attempts)

5. 1 to 3 min warm up before applying load

6. Apply load

a. Main (load circuit breakers) –OFF

b. Power selector switch – GEN

c. Main – ON

d. AC breakers – ON

COMMAND VEHICLE CHECKLIST

## POWER UP  with EXTERNAL POWER

1. Power selector switches – OFF

2. Shore cord pass-through door – OPEN

3. Shore cord connected to transformer

4. Primary AC volt meter circuit breaker – ON

5. External power voltage – MONITORED

6. Transformer primary tap –SELECTED/CONNECTED

7. Main Circuit Breaker/all distribution circuit breakers – OFF

8. Power Selector switch – EXT

9. Main Circuit Breaker at tap transformer – ON

10. At Power Distribution Panel:

a. Voltmeter/ammeter selector switch 
              *       line1 – CHECKED (120/125 v)

**    line2 – CHECKED (120/125 v) 
11. Apply load

a. Main (load circuit breakers) –OFF

b. Power selector switch – GEN

c. Main – ON

d. AC breakers – ON

COMMAND VEHICLE CHECKLIST

STABALIZER JACK OPERATION 

    EXTENSION

1. Power up checklist - COMPLETED

2. Stabalizer circuit breaker – ON

3. Ground obstructions - CHECKED

4. Power – ON

5. Left Stabalizer Jack – EXTENDED

6. Right Stabalizer Jack – EXTENDED

7. Jacks seated properly – CHECKED

RETRACTION

1. Stabalizer circuit breaker – ON

2. Ground obstructions - CHECKED

3. Power – ON

4. Left Stabalizer Jack – RETRACTED

5. Right Stabalizer Jack – RETRACTED

6. Jacks seated properly – CHECKED

COMMAND VEHICLE CHECKLIST

# MAST OPERATION  -  EXTENSION

1. Power up checklist - COMPLETED

2. Overhead obstructions – CHECKED

3. Stabalizing Jacks checklist– COMPLETED

4. Stabilizing jacks - EXTENDED

5. Mast Compressor

a. “COMPRESSOR” circuit breaker – ON

b. Mast control breaker – ON

6. Mast warning actuating pin – REMOVED

7. Overhead obstructions – CHECKED

8. Nycoil – INSPECTED

9. Mast control switch – RAISE POSITION

10. Mast extension observed for obstructions –OBSERVED

11. Mast fully extended – CHECKED

12. PSI at 16 psi (min) to 35 psi (max) - CHECKED

COMMAND VEHICLE CHECKLIST

MAST OPERATION (cont.)

### WARNING

Do not attempt to adjust mast pressure above 35 psi. Personal injury and/or serious mast damage could result

### WARNING

Serious injury or death could result if the mast or antennas come in contact with or near overhead wires

## NOTE

COMMAND VEHICLE CHECKLIST

MAST OPERATION   -    RETRACTION

1.  Obstructions on vehicle roof – CHECKED

2. Mast control switch – DOWN POSITION

3. Nycoil bundle – CHECKED

4. Mast warning actuator pin – INSTALLED

COMMAND VEHICLE CHECKLIST

## POWER DOWN GENERATOR

1. Under counter computer/radio switches - OFF

2. All circuit breakers – OFF

3.  “MAIN” circuit breakers – OFF

4. Power Selector switches – OFF

5. Generator – RUN FOR COOL DOWN  (3 TO 5 MIN)

6. Generator START/STOP switch – STOP (hold until generator at full stop

POWER DOWN EXTERNAL

1. All circuit breakers – OFF

2. “MAIN” circuit breakers – OFF

3.  Power Selector switches – OFF

4. Power Cords at source – DISCONNECTED

5. Power Cords at VEHICLE  – DISCONNECTED

COMMAND VEHICLE CHECKLIST

PRIOR TO DEPARTURE

1. Mast Retracted – CHECKED

2. Stabalizing jacks – RETRACTED

3.  Generator/External Power – OFF/REMOVED

4. Generator compartment door – LOCKED

5. Roof ladder – INSTALLED

6. Mast compartment door – LOCKED

7. All interior electrical power/devices – OFF

8. Loose interior equipment - SECURED

9. Main Ladder – STOWED

10. Main Door – LOCKED

11. Vehicle checked for overhead and underside obstructions/loose equipment – CHECKED

12. Chocks - REMOVED

COMMAND VEHICLE CHECKLIST

STABALIZING JACK OPERATION

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

COMMAND POST OPERATION CHECKLIST

Radios –ON

Telephone – ON

Computers – ON

Plasma TV - ON